11 February 2026

The Church Council

Immanuel Church Seatides

172-188 Genazano Road

Tongaat

Seatides

RE: BILL OF QUANTITIES – PROPOSED RENOVATIONS TO IMMANUEL CHURCH SEATIDES

Our Ref: ME-408

Dear Sirs

Please find enclosed our Provisional Bill of Quantities for the proposed renovations to Immanuel Church Seatides, located at Lot 6459 Tongaat, 172-188 Genazano Road.

This submission has been compiled based on the architectural drawings prepared by Narrative Architects and structural drawings prepared by Sutherland Engineers (Project No. 9721). A detailed breakdown of the scope of works, exclusions, provisional sums, and reference drawings is provided below for your consideration.

1. BREAKDOWN OF PRICE

The following is a complete breakdown of all trade sections included in this Bill of Quantities:

| Bill No. | Description | Amount (R) |
| --- | --- | --- |
| 01 | Preliminaries (Establishment, Site Overheads, Insurances, Site Toilets) | 604,500.00 |
| 1 | Alterations (Demolition, Wall Repairs, Crack Repairs, Doweling) | 173,742.50 |
| 2 | Earthworks (Excavation, Filling, Compaction, Soil Poisoning) | 36,922.00 |
| 3 | Concrete, Formwork & Reinforcement (Ground Beams, Pads, Slabs, Columns, Beams) | 345,110.64 |
| 4 | Masonry (115mm, 230mm & 260mm Brickwork, Lintels, Reinforcement) | 380,507.80 |
| 5 | Waterproofing (DPC, Membranes) | 7,400.00 |
| 6 | Carpentry & Joinery (Doors D04–D12, Frames, Skirtings) | 39,365.80 |
| 7 | Ceilings & Partitions (Suspended, Raked, Eaves Ceilings, Insulation, Drywall) | 215,422.90 |
| 8 | Floor Coverings (Berber Point 650 Carpet Tiles – 50cm x 50cm) | 98,000.00 |
| 9 | Plastering (Internal/External Plaster, Screeds, Decorated Screeds) | 293,031.00 |
| 10 | Tiling (Floor Tiling at R250/m² PC Rate) | 40,880.00 |
| 11 | Plumbing & Drainage (WHB, WC, Sink, Prep Bowl, Geysers, Connections) | 51,450.00 |
| 12 | Paintwork (Walls, Ceilings, Wood Surfaces, Plasterboard) | 187,429.10 |
| 13 | Subcontract Packages (See Section 3 for detailed breakdown) | 1,352,182.06 |
| 14 | Provisional Sums (See Section 4 for detailed breakdown) | 175,000.00 |
| Ext | External Works – Walkways | 9,620.00 |
| Ext | External Works – Stormwater | 33,013.44 |
|  | Total Excluding VAT | R 4,043,577.24 |
|  | Markup (7.5%) | R 303,268.29 |
|  | Total Including Markup | R 4,346,845.53 |
|  | VAT (15%) | R 652,026.83 |
|  | TOTAL INCLUDING VAT | R 4,998,872.36 |

2. EXCLUSIONS

The following items are specifically EXCLUDED from this Bill of Quantities and have NOT been priced:

Soak Pit – As noted on the drawings, there is no detail provided for any soak pit. This item has been excluded pending clarification of engineering details.

All Moveable Furniture – No allowance made for any loose or moveable furniture items.

All Sound Equipment – No audiovisual or sound system equipment has been included.

All Curtains and Blinds have been excluded.

Backup Power and Solar Installations – No provision for generators, UPS systems, or solar panels.

Engineering Fees – Professional engineering fees are excluded.

Architectural Fees – Professional architectural fees are excluded.

Sandpit and Jungle Gym Installations – No work priced for sandpit and jungle gym relocations or installations.

Air Conditioning Work – No allowance made for any air conditioning work, whether new installations or relocation of existing units. Any work in relation to HVAC, air conditioning, or extraction needs to be confirmed and communicated prior to pricing.

Security Systems – No alarm or security system installations have been included.

IT Systems – Only a provision for the sound deck and stage physical installations has been made, with a provisional allowance for conduits and sleeves for future sound cabling to be installed by others. No IT infrastructure has been included.

IMPORTANT NOTE REGARDING LOWER LEVEL WORKS:

No work has been included to the Lower Level with the following exceptions:

New Youth/Multi-Purpose Room construction and finishes

Complete repaint of Lower Level internal walls including preparation and crack repairs

All other aspects of the Lower Level remain unchanged and are NOT covered in this submission:

All floor finishes remain as existing

All kitchens remain as existing

All bathrooms remain as existing

All storerooms remain as existing

All windows and doors on the Lower Level remain as existing, unless specifically appearing on the Window and Door Schedule and related to the Multi-Purpose Room

3. SUBCONTRACT PACKAGES

The following Subcontract Packages have been included in this submission. These packages represent specialist trade works that have been priced based on subcontractor proposals or estimated allowances (the detail and the specifications for the below packages are included in this section below). Final pricing will be subject to confirmation of scope details (should they be changed by the Client) and receipt of firm quotations where applicable.

| No. | Item Description | Allowance (ex VAT) |
| --- | --- | --- |
| 1a | Electrical Installation | R 287,405.00 |
| 1b | Three Phase Upgrade – Optional | R 46,400.00 |
| 2 | Timber Staircase and Timber Deck | R 103,655.00 |
| 3 | Aluminium Windows and Doors | R 302,123.93 |
| 4 | Roof Supply and Installation | R 477,690.60 |
| 5 | Metal Palisade/Gate (GMS Screen) | R 38,510.60 |
| 6 | Balustrades | R 17,556.93 |
| 7 | Rainwater Goods | R 29,700.00 |
| 8 | Timber Screen | R 49,140.00 |

Electrical Installation (Items 1a and 1b)

The electrical installation has been priced based on a quotation received from CW Electrical. The scope includes all new electrical reticulation for the additions, including distribution boards, circuit wiring, light points, plug points, switches, and external lighting as indicated on the architectural drawings. Light fittings are included in the price. An optional three-phase power upgrade has been priced separately at R46,400.00 (ex VAT) for the Church’s consideration – this would involve upgrading the existing single-phase supply to three-phase to accommodate future loading requirements including potential audio-visual and HVAC (Air-conditioning) systems. The three-phase upgrade is included in the base total and should this not be required, this amount will be deducted from the total.

Summarized details of this package:

| Description | Unit | Qty |
| --- | --- | --- |
| UPPER GROUND STOREY - LINE ITEM BREAKDOWN |  |  |
| KITCHEN |  |  |
| Single plug point - fridge | No | 1 |
| Single plug point - MW | No | 1 |
| 40A isolator point - industrial coffee machine | No | 1 |
| Double plug points - above counter | No | 3 |
| Three pin / two pin combo plug point | No | 1 |
| Stove isolator point | No | 1 |
| Connect / fit oven and hob | Item | 1 |
| Recessed light points | No | 4 |
| 10W Legend round LED downlighter fittings | No | 4 |
| Ceiling fan point - wiring / assemble and install | No | 1 |
| Pels 1400mm three blade ceiling fan with remote | No | 1 |
| Light switch point | No | 1 |
| FOYER |  |  |
| Double plug point | No | 1 |
| Three pin / two pin combo plug point | No | 1 |
| Recessed light points | No | 6 |
| 10W Legend round LED downlighter fittings | No | 6 |
| Light switch point | No | 1 |
| LOUNGE AREA ADJACENT TO FOYER |  |  |
| Double plug points | No | 3 |
| Three pin / two pin combo plug points | No | 2 |
| Recessed light points | No | 8 |
| 10W Legend round LED downlighter fittings | No | 8 |
| Ceiling fan points - wiring / assemble and install | No | 2 |
| Pels 1400mm three blade ceiling fans with remotes | No | 2 |
| Light switch point | No | 1 |
| MAIN AUDITORIUM |  |  |
| Double plug points - general use | No | 4 |
| Three pin / two pin combo plug points - general use | No | 2 |
| Double plug points - sound & media station | No | 6 |
| Double plug points - main stage | No | 8 |
| Recessed light points - main double volume | No | 32 |
| 35W Legend round LED downlighter fittings | No | 32 |
| Recessed light points - main stage | No | 12 |
| 10W Legend round LED downlighter fittings | No | 12 |
| Main light switch point (two banks of 16) | No | 1 |
| Two way switching point | No | 1 |
| STORAGE ROOM 1 & 2 |  |  |
| Recessed light points - main stage | No | 4 |
| 10W Legend round LED downlighter fittings | No | 4 |
| Light switch points | No | 2 |
| PRAYER ROOM |  |  |
| Double plug point | No | 1 |
| Three pin / two pin combo plug point | No | 1 |
| Recessed light points | No | 4 |
| 10W Legend round LED downlighter fittings | No | 4 |
| Ceiling fan point - wiring / assemble and install | No | 1 |
| Pels 1400mm three blade ceiling fan with remote | No | 1 |
| Light switch point | No | 1 |
| MOTHERS ROOM |  |  |
| Double plug point | No | 1 |
| Three pin / two pin combo plug point | No | 1 |
| Recessed light points | No | 4 |
| 10W Legend round LED downlighter fittings | No | 4 |
| Ceiling fan point - wiring / assemble and install | No | 1 |
| Pels 1400mm three blade ceiling fan with remote | No | 1 |
| Light switch point | No | 1 |
| TOILET / SMALL PASSAGE |  |  |
| Recessed light points | No | 3 |
| 10W Legend round LED downlighter fittings | No | 3 |
| Light switch point | No | 1 |
| EXTERNAL STAIRCASE |  |  |
| Outside wall light points | No | 2 |
| Hero Pioled black trim square LED bulkhead light fittings | No | 2 |
| Photocell point | No | 1 |
| OPEN DECK & EXTERNAL - LINE ITEM BREAKDOWN |  |  |
| OPEN DECK ARRIVAL |  |  |
| Outside wall light point - ramp walkway | No | 1 |
| Ground light points - feature either side of main door (wooden decking) | No | 2 |
| Ground light points - feature signage wall (wooden decking) | No | 3 |
| Ground light points - wooden deck steps/stairs x 3 | No | 6 |
| Outside wall light point - stairs | No | 1 |
| Hero Pioled black trim square LED bulkhead light fittings | No | 2 |
| Spazio round black 5W LED ground light fittings | No | 11 |
| AIRCONDITIONING - MAIN AUDITORIUM (PROVISIONAL) |  |  |
| 40A single phase aircon isolator points - separate cable runs from DB incl D-curve circuit breakers at external condenser units (cassette type AC ceiling mount) | No | 4 |
| Single plug points - indoor cassette units | No | 4 |
| SOUND / AUDIO VISUAL (PROVISIONAL) |  |  |
| 32mm conduit links: main stage to sound/media control - sound | Item | 1 |
| 32mm conduit links: main stage to sound/media control - audio visual | Item | 1 |
| 32mm conduit links: main stage to sound/media control - network | Item | 1 |
| 32mm conduit links to speaker points - positioned around main auditorium | No | 6 |
| 32mm conduit link from visual screen to sound/media control | Item | 1 |
| Power point - main screen | No | 1 |
| GENERAL / DISTRIBUTION BOARD |  |  |
| 3x12 way flush DB with door incl all relevant switchgear (kitchen behind door) - sized for future 3-phase upgrade | No | 1 |
| 16mm x 2 core ECC cable run from meterbox (ground floor) to new DB (kitchen) incl conduits/chasing/fixings | Item | 1 |
| Geyser isolator point - upper level kitchen/toilet | No | 1 |
| CBI Range electrical sockets and light switches (all listed above) | Item | 1 |
| LOWER GROUND STOREY - LINE ITEM BREAKDOWN |  |  |
| YOUTH / MULTI-PURPOSE ROOM |  |  |
| Double plug points | No | 3 |
| Three pin / two pin plug points | No | 2 |
| TV point incl single plug @ 1.8m high on wall - TV screen | No | 1 |
| Recessed light points incl concrete slab boxes | No | 10 |
| 10W Legend round LED downlighter fittings | No | 10 |
| Ceiling fan points - wiring / assemble and install | No | 2 |
| Pels 1400mm three blade ceiling fans with remotes | No | 2 |
| Outside wall light points | No | 2 |
| Hero Pioled black trim square LED bulkhead light fittings | No | 2 |
| Light switch point | No | 1 |
| Aircon isolator point - separate cable feed from new DB upper incl D-curve circuit breaker - condenser unit (18000 BTU) | No | 1 |
| EXISTING AREAS LOWER GROUND (PROVISIONAL) |  |  |
| Inspect and test existing electrical points and wiring for water damage; ensure compliance with electrical regulations; advise client on repairs required and costs | Item | 1 |
| TRAVEL |  |  |
| Site travel costs | Item | 1 |
| THREE PHASE UPGRADE - PROVISIONAL BREAKDOWN |  |  |
| 16mm x 4 core plus ECC cable - meterbox to DB upper level | Item | 1 |
| Three phase meterbox - lower level | No | 1 |
| Additional switchgear (note: 3-phase DB size already allowed for in main quote) | Item | 1 |
| Three phase upgrade approx cost - Durban Metro Electricity Department | Item | 1 |

Timber Staircase and Timber Deck (Item 2)

This package covers the supply and installation of the new timber staircase and timber arrival deck as indicated on the drawings. The scope is based on a specialist subcontractor proposal and includes all structural timber members, fixings, to the deck area.

Summarized details of this package:

| Item | Description |
| --- | --- |
| 1. DECKING - SPECIFICATION & INCLUSIONS |  |
| BALAU DECK (24 m²) |  |
| 1.1 | Install new Balau 24.081 m² deck as per plans with two steps on the front |
| 1.2 | Decking: Imported 19/90mm Balau laid with 5mm spacings between boards |
| 1.3 | Timber source: iDeck timber - high quality, harvested from sustainable forest management plantations |
| 1.4 | Substructure beams: H4 CCA treated pine, 228/50mm wide (wider beams for longevity and reduced board breakage at joins) |
| 1.5 | Substructure joists: H4 CCA treated pine, 152/50mm wide (deepest possible joist to eliminate deck bounce) |
| 1.6 | Vertical supports: H4 CCA treated gum poles secured into concrete footings |
| 1.7 | All substructure below deck painted black (to hide structure in shadows below) |
| 1.8 | Two-deck board fascia hiding deck structure for neat and clean finish |
| 1.9 | Timber sanded and oiled with Woodoc deck dressing |
| 2. BALAU STAIRS - SPECIFICATION & INCLUSIONS |  |
| BALAU STAIRS (17 treads) |  |
| 2.1 | Build 17 tread Balau stairs as per plans |
| 2.2 | Treads: 2 x 140/30mm Balau with 5mm space between, secured onto 40/40mm side cleats |
| 2.3 | Tread planks centrally secured together by single board |
| 2.4 | Stringers: 215/40mm Balau |
| SUPPLIER INCLUSIONS |  |
| • All materials, fixings and epoxies where needed |  |
| • All timber is iDeck - high quality from sustainable forest management plantations |  |
| • Woodoc deck dressing included for sanding and oiling |  |
| • All substructure painting (black) included |  |
| • Professional installation |  |

Aluminium Windows and Doors (Item 3)

This package covers the supply and installation of all new aluminium windows, doors, as detailed in the Window and Door Schedules (NA_2407_800 and NA_2407_810) – Note that the Window and door schedules are not entirely accurate as it relates to the floor plans and for this reason we have taken time to review each of the door and window requirements and tabled (below). The price of R302,123.93 (ex VAT) is based on our subcontractor pricing and includes all units as reconciled above.  Any windows or doors on the existing Lower Level are excluded unless specifically referenced in the schedules in relation to the Multi-Purpose Room.

Detailed cost breakdown for reference purposes:

| Ref | Qty | Description | Size (WxH) | Price/Item | Total (ZAR) |
| --- | --- | --- | --- | --- | --- |
| WINDOWS - LINE ITEM BREAKDOWN |  |  |  |  |  |
| GENERAL |  |  |  |  |  |
| Powder Coating | 1 | Cost for D2525 powder coating for coastal areas (recommended for dark colours) |  | R 18,000.00 | R 18,000.00 |
| FOYER WINDOWS |  |  |  |  |  |
| W01 - Foyer | 1 | Shopfront window - 4 fixed panes + R4 | 2150 x 3000 | R 13,142.86 | R 13,142.86 |
| W02 - Foyer (Opt 1) | 1 | Shopfront window - 3 fixed panes - no R4 | 2700 x 2100 | R 11,142.86 | R 11,142.86 |
| W03 - Foyer | 1 | 340 Casement window - 1 top hung above 1 fixed pane - no R4 | 900 x 2100 | R 4,300.00 | R 4,300.00 |
| AUDITORIUM WINDOWS |  |  |  |  |  |
| W04 - Auditorium | 1 | Shopfront window - 6 top hung above 3 fixed panes - no R4 | 1800 x 2100 | R 17,142.86 | R 17,142.86 |
| W05 - Auditorium | 1 | Shopfront window - 6 top hung above 3 fixed panes - no R4 | 1800 x 2100 | R 17,142.86 | R 17,142.86 |
| W08 - Auditorium | 1 | Shopfront window - 2 top hung above 1 fixed pane on left of 1 fixed pane with fixed toplight - no R4 | 1620 x 2700 | R 12,714.29 | R 12,714.29 |
| PRAYER ROOM / WC WINDOWS |  |  |  |  |  |
| W06 - Prayer Room | 1 | 340 Casement window - 2 top hung above 1 fixed pane | 900 x 1800 | R 4,533.33 | R 4,533.33 |
| W07 - Disabled WC | 1 | 340 Casement window - 1 top hung above 1 fixed pane | 400 x 1800 | R 2,416.67 | R 2,416.67 |
| MULTIPURPOSE ROOM WINDOWS |  |  |  |  |  |
| W09 - Multipurpose | 1 | 340 Casement window - 1 top hung on both sides of 1 fixed pane | 3000 x 600 | R 5,166.67 | R 5,166.67 |
| W10 - Multipurpose | 1 | 340 Casement window - 1 top hung on both sides of 1 fixed pane | 3000 x 600 | R 5,166.67 | R 5,166.67 |
| W11 - Multipurpose | 1 | 340 Casement window - 2 top hung above 1 fixed pane | 900 x 1800 | R 4,533.33 | R 4,533.33 |
| DOORS - LINE ITEM BREAKDOWN |  |  |  |  |  |
| FOYER DOORS |  |  |  |  |  |
| D01 - Foyer | 1 | Double door with stack sections, parliament hinges, fixed sidelight on both sides and 2 fixed toplights | 3000 x 3000 | R 25,714.29 | R 25,714.29 |
| COVERED VERANDAH DOORS |  |  |  |  |  |
| D02 - Cvd Verandah | 1 | Double door with stack sections, parliament hinges, fixed sidelight on both sides and 2 fixed toplights | 3000 x 3000 | R 25,714.29 | R 25,714.29 |
| D03 - Cvd Verandah | 1 | 6 pane Palace sliding door centre closing | 8000 x 2400 | R 54,068.97 | R 54,068.97 |
| D05 - Cvd Verandah | 1 | Single door with stack sections, parliament hinges, sidelight on left with 2 top hung above 1 fixed pane + R4 & toplight with 2 fixed panes | 1430 x 3300 | R 19,285.71 | R 19,285.71 |
| WALKWAY / MULTIPURPOSE DOORS |  |  |  |  |  |
| D11 - Walkway | 2 | Double door with stack sections, parliament hinges, fixed sidelight on both sides + R4 | 3458 x 2100 | R 21,428.57 | R 42,857.14 |
| D13 - Multipurpose | 1 | Double door with stack sections, parliament hinges | 1800 x 2100 | R 13,571.43 | R 13,571.43 |
| ADDITIONAL ITEMS |  |  |  |  |  |
| Waterproofing Detail | 1 | Fitment of aluminium angles beneath all windows and doors at finished floor level prior to waterproofing done by others |  | R 5,279.73 | R 5,279.73 |
| Glazing Certificate | 1 | Cost of glazing certificate provided upon full settlement of account |  | R 230.00 | R 230.00 |
| PRICING SUMMARY |  |  |  |  |  |
| SUBTOTAL (excl VAT) |  |  |  |  | R 302,123.93 |

Roof Supply and Installation (Item 4)

The roof package covers the complete supply and installation of the new Timber Only roof structure.   , roof sheeting, all associated structural timber members, and fascia boards. The scope aligns with the structural drawings which indicate “Timber Roof Structure by Specialist Design, Supply, and Install”, but no steel members have been allowed for.  The price of R477,690.60 (ex VAT) is inclusive of the specialist roof truss design, manufacture, delivery, and erection. Roof sheeting profile and colour are to be confirmed by the Architect.

Metal Palisade/Gate – GMS Screen (Item 5)

This item covers the supply and installation of the galvanised mild steel (GMS) screen/palisade as indicated on the architectural drawings. The price of R 38,510.60 (ex VAT) is based on a subcontractor proposal for the manufacture and installation of the palisdae complete with gate and all fixings.

Balustrades (Item 6)

This item covers the supply and installation of the galvanised mild steel (GMS) Balustrade, powder coated black to new timber staircase, portion of deck and also top of existing stair – total meterage = 9m. The price of R 17,556.93 (ex VAT) is based on a subcontractor’s rates included in the proposal for the manufacture and installation of same.

Rainwater Goods (Item 7)

This package covers the supply and installation of all new gutters, downpipes, and connections to the stormwater system as indicated on the architectural and structural drawings. The price of R29,700.00 (ex VAT) includes all materials, fixings, and labour for the complete rainwater goods installation to the new roof areas.Price is based on 74m of gutter and 43m of downpipes.

Timber Screen (Item 8)

The architectural drawings reference “repurposed existing timber” for the timber screen element. However, there is no repurposed existing timber available on site. This item has therefore been priced as a new timber screen (Balau) at a price of installation at R49,140.00 (ex VAT) based on a subcontractor’s proposal for the supply, manufacture, and installation of a new screens (2 off) to the Architect’s design intent.

Additional Scope Notes – Floor Coverings and Tiling

Floor Coverings – Carpet (Bill 8)

The floor covering price of R98,000.00 (ex VAT) is based on the supply and installation of Berber Point 650 carpet tiles in a 50cm x 50cm format to an area of approximately 261m². This specification has been selected for its durability and suitability for a church environment with high foot traffic. The rate is inclusive of supply, delivery, and installation complete with all necessary adhesive. The Client should note that this is a specified product and any changes to the carpet specification or product selection may result in a price adjustment.

Floor Tiling (Bill 10)

Floor tiling has been measured to an area of approximately 73m² and priced at a Prime Cost (PC) rate of R250/m² for the tile supply only. The all-in rate of R560/m² includes for taking delivery, storage, fixing with adhesive to screed (screed measured separately under Plastering), and flush pointing with tinted waterproof grout. The PC rate of R250/m² is a provisional allowance only – the final tile selection and associated cost will be determined by the Client. Any difference between the PC rate and the actual cost of tiles selected will be adjusted on the final account.

4. PROVISIONAL SUMS

The following Provisional Sums have been included in this priced submission. These items are provisionally included for finalization with the Client during the contract period as there are not sufficient details around these – they are essentially place holders for these items at realistic budgets. Final pricing will be based on Client selections and specialist quotations.

| No. | Item Description | Purpose/Notes | Allowance (ex VAT) |
| --- | --- | --- | --- |
| 1 | Ironmongery | Door handles, locks, hinges – based on Client selection | R 15,000 |
| 2 | Landscaping | Minor landscaping allowance | R 25,000 |
| 3 | Sanware, Brassware & Geysers | Toilets, wash basins, taps, geysers, gas geyser etc. | R 30,000 |
| 4 | Mirrors & Bathroom Accessories | Bathroom fixtures and accessories | R 5,000 |
| 5 | Signage | General signage allowance | R 10,000 |
| 6 | Sewer | Sewer connections and works | R 15,000 |
| 7 | Feature Wall Signage | Feature signage element | R 5,000 |
| 8 | Stage | Stage construction and installation | R 40,000 |
| 9 | Joinery for Sound and Media | Joinery and built-in elements for sound/media installation | R 30,000 |

IMPORTANT NOTE REGARDING PROVISIONAL SUMS:

The above Provisional Sums are estimates only and have been included to ensure these items are accounted for within the project budget. The amounts listed do not represent confirmed prices or any specific product selections by the Church. No selections have been made by the Client at this stage and the provisional amounts carry no correlation to the final costs that will be incurred. These sums will be adjusted on the final account based on actual costs arising from Client selections, specialist quotations, and confirmed scope. The Client is encouraged to engage with the relevant suppliers and specialists as early as possible to allow for accurate pricing and to avoid delays during the construction period.

5. REFERENCE DRAWINGS

This Bill of Quantities has been compiled using the following drawings:

ARCHITECTURAL DRAWINGS (Narrative Architects):

NA_2407_310 – Upper Ground Storey Plan

NA_2407_300 – Lower Ground Storey Plan

NA_2407_320 – Roof Storey Plan

NA_2407_400 – Sections and Elevations

NA_2407_800 – Window & Door Schedule 01

NA_2407_810 – Window & Door Schedule 02 (including General Specifications and Finishes)

STRUCTURAL DRAWINGS (Sutherland Engineers – Project No. 9721):

9721_S_DW001 – Foundation Layout and Details

9721_S_DW002 – Surface Bed Layout

9721_S_DW003 – First Floor and Roof Layout

9721_S_DW004 – Building Sections

9721_S_DW005 – Typical Brickwork Details

PROJECT SPECIFICATION:

9721 Project Specification (Sutherland Engineers) – Revision P01, dated 28 July 2025

6. NOTES AND CONDITIONS

This is a Provisional Bill of Quantities and is subject to adjustment based on final design confirmation and site conditions.

All Provisional Sums are subject to finalization with the Client during the contract period.

Quantities have been measured from the drawings listed above. Any discrepancies between drawings should be reported to the Architect for clarification.

PC (Prime Cost) rates (i.e. Tiling specification) are provisional and subject to final selection by the Client.

All work to comply with SANS 10400 regulations and relevant South African building codes.

Contract Works Insurance for existing building is excluded – to be arranged by Client.

Payment Terms:  Upon appointment an establishment Fee of R 200,000 + Vat is payable, whereafter monthly draws will be submitted on the 20th of each month, which will then be payable 7 calendar days later.

We trust this submission meets with your approval and look forward to your favourable response.

Should you require any clarification or further information, please do not hesitate to contact the undersigned.

Yours faithfully

_______________________________

Simon Strang

For and on behalf of DF Strange Buildings